# To Bucher

- Get the assignments instructions and grading out of the powerpoints (to avoid inconsistency)

- Go over my plan: can I continue planning?

- IOC – will they build or get one? (I think they should build, not so much of the IO, but for the uart \ encoder settings)

- I2C – cosider leave it as a black box? Maybe only a minimal “is device there” check..

# Questions to claude:

- Les2 s18. It shows: reg = (reg & ~MODE_MASK) | (2u << MODE_POS);

1) clear the field    2) write new value

Why is it needed? Cant we set it in the style of slide 17, as such:

reg |=  (3u << 4); ??

- Les2 s36 : “Indent consistently (CubeIDE: Ctrl+Shift+F) — unreadable code is ungraded code” what does it mean? What is indent?

# Already answered questions:

- In les2 s8, you say “smaller is not automatically better”. Why is that?

The Cortex-M4 is a 32-bit CPU: its registers and ALU are 32 bits wide. Any arithmetic on a uint8_t or int16_t is first promoted to a 32-bit int, done in 32 bits, and then truncated back when it is stored, which can cost an extra instruction (UXTB/SXTH) and silently wraps around. The only gain of a small type is memory, which matters for large arrays and buffers, or when the size must match the hardware (an I2C byte, an 8-bit register). Classic trap: for (uint8_t i = 0; i < 300; i++) never ends, because i wraps from 255 to 0. Hence the slide's rule: small types to store data, int32_t/uint32_t for counters and arithmetic.

- Why use float to speeds, meters? Isn’t it better to use scaled integers only?

Scaled integers (fixed point) are the right choice on an MCU without an FPU. The G474 has a hardware single-precision FPU, so a float add or multiply takes about one cycle, as cheap as an integer one, and float avoids the scaling bugs (overflow, lost precision, mixed-up scale factors). Three cautions: (1) always write the f suffix (0.5f); 0.5 is a double, and double is emulated in software, so it is slow. (2) float has about 7 significant digits, so do not accumulate time or encoder counts in a float; keep counts as integers and convert to float only for the physics (speed = counts / dt). (3) Things that must be exact (counters, timestamps, register values) stay integers.

C: But, is it a reasonable thing to say that in some applications, there is advantage to work only with integers and not floats? And to give a part of an assignment with a strict rule  that they must not use float?

Yes, and it is a good assignment item. Integer-only code is the right choice when: (1) the MCU has no FPU (Cortex-M0/M0+, e.g. STM32G0, the chips in most cheap products), where every float operation is emulated in software and costs tens of cycles; (2) code runs in a fast interrupt and must take a fixed, known time; (3) results must be exact (counters, comparisons with ==, money); (4) DSP work in fixed point (Q15/Q31 formats, as in CMSIS-DSP). A rule such as "this part without float" forces students to choose a scale (e.g. mm/s instead of m/s, or Q16.16), to think about overflow (an int32_t intermediate, an int64_t for products) and rounding, which is types, wrap-around and shifting in one exercise. A strong version: implement the same computation (wheel speed, or the PI update) in float and in fixed point, compare the results, and measure the cycle count of each with DWT->CYCCNT.

- Les2 s9 : “uint8_t duty = (uint8_t)(ratio * 255.0f);   “ what is this?

ratio is a float between 0.0 and 1.0 (fraction of full brightness or power). Multiplying by 255.0f gives a float between 0.0 and 255.0, and the cast (uint8_t) converts it to a byte, the 0..255 value the PCA9634 PWM register expects. The explicit cast says "I know I am throwing away the fraction". Notes: the cast truncates, it does not round (254.9 becomes 254; add 0.5f to round). If ratio is outside 0..1, converting an out-of-range float to an integer type is undefined behaviour in C, so clamp ratio first.

- Les2 s13: “Convention: #define for masks/sizes, const for physical quantities with units”   ??

#define is plain text substitution by the preprocessor: no type, no memory. It suits masks and sizes because in C (unlike C++) a const variable is not a compile-time constant, so it cannot size a static array or appear in a case label, while a #define can. const float WHEEL_R_M = 0.030f is a real, typed, read-only variable: the compiler type-checks it, the debugger can show it, and the name carries the unit (_M = metres). So the convention means: integer constants that the compiler needs at compile time go in #define; physical parameters go in const. It is a style rule, not a language rule.

C: But still, cant I write something like “define WHEEL_R_M 0.03”, and then use this definition in any calc that requires this radius? Thus, for example, If I later have a cart with a radius of 0.04, I can simply change the value in the #define, exactly as done with N_LEDS. I cant see the essential difference between the two. Both can be written as “#define” or as a variable assignment.

You are right: #define WHEEL_R_M 0.03f works, and the one-place-to-change benefit is identical. The differences are secondary, which is why it is only a convention: (1) Type: 0.03 without the f is a double, so every calculation using it is promoted to double, which is emulated in software and slow; a const float fixes the type. (2) Debugger: a const is a real variable you can see by name; a macro disappears after the preprocessor. (3) Scope: a macro is global from the line it is defined and ignores braces and files, so names can clash; a (static) const obeys scope. (4) Expression trap: #define R 0.02f+0.01f makes 2*R equal 0.05, not 0.06, unless you wrap it in parentheses. Going the other way, a const cannot size an array or be a case label in C. Many embedded code bases use #define for everything; that is fine if every value has its suffix and its parentheses.

- Les2 s17:  k << 1      /* k * 2 */        k >> 3      /* k / 8 (unsigned)   */  ??

In binary every position is worth twice the one to its right, so shifting left by one place multiplies by 2 (like appending a 0 in decimal multiplies by 10): k << n = k * 2^n. Shifting right by 3 divides by 2^3 = 8 and drops the remainder (13 >> 3 = 1). "(unsigned)" is a warning: for a negative signed number, right shift is implementation-defined (on ARM GCC it is an arithmetic shift that rounds toward minus infinity, so -1 >> 3 = -1 while -1 / 8 = 0), and left-shifting a negative or overflowing signed value is undefined. In practice: the compiler already turns k / 8 into a shift when k is unsigned, so write / and * for arithmetic and use shifts when you mean bits.

C: so for a signed -1. If I do >>3. Would it be -1 or 0? Write me first the binaric rep of -1, than it bitshifts, and explain how it derives the final result (of 0 or =1 or whatever)

It is -1. Take int8_t for clarity. +1 is 00000001; two's complement of it: invert the bits, 11111110, and add 1, 11111111. That is -1. A signed right shift on ARM GCC is arithmetic (the ASR instruction): every bit moves one place to the right, and the empty place on the left is filled with a copy of the sign bit, so the sign is kept. 11111111 >> 1 = 11111111, again and again, so -1 >> 3 = 11111111 = -1. (An unsigned, logical shift fills with 0s instead: 11111111 >> 3 = 00011111 = 31.) Arithmetic shift rounds toward minus infinity: -1/8 = -0.125, floor = -1. C division truncates toward zero: -1 / 8 = 0. So the shift gives -1 and the division gives 0. Another example: -9 = 11110111; >> 3 = 11111110 = -2 (floor of -1.125), while -9 / 8 = -1.

- Les2 s17: On the cart: (3 << i) walks a 2-LED pattern along the 8 LEDs — you will use it in lesson 3

3 is 0b00000011, two adjacent bits set. (3 << i) moves that pair to bits i and i+1. Each bit of the 8-bit LED mask drives one LED (bit n = LED n), so in a loop i = 0..6 the lit pair walks along the strip: LEDs 0+1, 1+2, ..., 6+7. At i = 7 the upper bit (bit 8) falls off an 8-bit mask, and only LED 7 stays lit. To make the pattern wrap around: mask = ((3u << i) | (3u >> (8 - i))) & 0xFF for i = 1..7.

- Les2 s 17: explain me : 1u << n     /* THE mask builder: a 1 at bit n                    */

1u << 7

1u is the number 1 as an unsigned int: binary ...00000001. Shifting it left by n places moves that single 1 to bit n: 1u << 0 = 0b00000001 (1), 1u << 3 = 0b00001000 (8), 1u << 7 = 0b10000000 (0x80 = 128). A number with 1s only at the bits you care about is called a mask, and you combine it with the bit operators: x |= (1u << n) sets bit n, x &= ~(1u << n) clears it, x ^= (1u << n) toggles it, and (x & (1u << n)) tests it. That is why the slide calls it THE mask builder: one expression gives the mask for any bit, and it reads better than a magic 0x80. HAL does the same: GPIO_PIN_5 is defined as 0x0020, which is 1u << 5. The u matters: plain 1 is a signed int, and 1 << 31 overflows a signed int, which is undefined behaviour; 1u << 31 is fine.

C: in x |= (1u << n) , what is “-=”?

The operator is |= (a vertical bar, not a minus): bitwise-OR assignment. x |= m means x = x | m: every bit that is 1 in m becomes 1 in x, and all other bits of x stay as they were. Example: x = 0b0101; x |= (1u << 1) gives 0b0111. It belongs to the same family as +=, -=, &=, ^=, <<=; for example x -= 3 means x = x - 3.

In  x &= ~(1u << n) , what is “&=”?

in x ^= (1u << n) what is “^=”

- Les2 s 17: it says “(v >> 4) & 0x0F   /* extract the upper nibble of v               .

What is nibble? Why is the part of “&0x0F”. if I write only (v>>4), isn’t it the same?

A nibble is 4 bits: half a byte, and exactly one hex digit. In v = 0xA7 the upper nibble is A (1010) and the lower nibble is 7 (0111). v >> 4 moves the upper nibble down to bits 0 to 3. If v is a uint8_t, you are right: (v >> 4) alone already gives 0x0A, and the & 0x0F changes nothing. The mask is there for the cases where it does matter: (1) v is wider than 8 bits: for uint16_t v = 0x12A7, v >> 4 = 0x12A, and only & 0x0F leaves 0xA, so the same expression extracts bits 4 to 7 of any width. (2) v is signed and negative: int8_t v = -16 (0xF0) is promoted to a 32-bit int 0xFFFFFFF0, and >> 4 copies the sign bit in, giving 0xFFFFFFFF (-1) instead of 0x0F; the mask fixes it. So it is a robustness habit, and it costs nothing: when it is redundant, the compiler removes it.

C: so it doesn’t take the uppr nibble. In the example of uint16_t v = 0x12A7, it take A, which is the “middle” nibble. Also you say “If v is a uint8_t, you are right: (v >> 4) alone already gives 0x0A”. Why is that? Who said the in the uppernibble the value was a?

Correct, and my wording was loose. "Upper nibble" assumes v is a byte: for 8 bits it means bits 4 to 7. (v >> 4) & 0x0F always takes bits 4 to 7, so for a 16-bit v = 0x12A7 it gives A, the second nibble from the right, not the top one. To get the top nibble of a 16-bit value you need (v >> 12) & 0x0F, which gives 1. As for "who said it was A": I was using my own example byte v = 0xA7. In general, for any uint8_t written as 0xXY, v >> 4 moves X down to bits 0 to 3 and fills bits 4 to 7 with zeros (unsigned shift), so the result is 0x0X, the upper nibble and nothing else. That is why the mask adds nothing for a uint8_t.