# Microprocessor-Based Product Design 2026/27
Course plan: sessions and assignments

Yohai Ackerman (TA), draft for discussion with Prof. Izhak Bucher

Draft of 2026-10-19

# Sessions

| # | Lecture date | Lecture | Tutorial | Milestone |
| --- | --- | --- | --- | --- |
| 1 | 2.11.2026 | Intro and product mindset; logic gates, flip-flops, counters, edge vs. level; integers in binary/hex, two’s complement, overflow. | First STM32 application: project from a provided .ioc → build → flash, blink LD2, printf to PuTTY, breakpoints and live expressions. |  |
| 2 | 9.11.2026 | Python → C: types (stdint), casting, scope, #define/const, operators and precedence, shifts and masks, pointers, structs, registers as pointers. | Numbers and bits on the board: predict-then-run wrap-around, signed/unsigned in the debugger, binary printer, the four bit idioms (set/clear/toggle/test). | A1 out: variables and wrap, bit operations, trigerring, timing, IR sensor logics |
| 3 | 16.11.2026 | The IO board as a black box: RGB LEDs (3 chips × 8 channels), colour mixing, gamma, patterns as data; buttons (active-low byte), edge detection, debouncing; “Catch the Dot”. Timers and timing: HAL_Delay vs. HAL_GetTick, non-blocking multi-rate timing. The UART protocol: baud rate, the 8N1 frame (start bit, 8 data bits, stop bit), the time to send one character. [Yohai to discuss with Bucher: move timers, HAL_GetTick and the UART protocol into lecture 3, since Assignment 1 needs them; the blue part could shrink to make room.] | RGB and buttons: set_rgb helper, walking dot with masks, saturating brightness up/down, one-press-one-action. | Last material included in A1 |
| 4 | 23.11.2026 | Timers: counter, prescaler, period, polling vs. interrupts, priorities, volatile; software state machines; HAL_GetTick non-blocking timing. Interrupts in depth: the interrupt service routine (ISR), the NVIC and the vector table, the HAL chain (IRQHandler → HAL_TIM_IRQHandler → callback), the cost of entering and leaving an interrupt, keeping ISRs short (no blocking calls, no HAL_Delay), variables shared with the main loop. | PWM, three methods: polling (HAL_GetTick in the main loop), interrupt (TIM7 callback) and hardware (timer CCR); what each must fit in time; the ISR and the HAL callback chain. | Quiz 1 (first hour) |
| 5 | 30.11.2026 | The I2C bus opened: protocol, addressing, ACK/NACK, pull-ups; HAL I2C; register-level drivers for PCA9634, PCA9538A, PCA9685 (servo pulses). | I2C bus scan: detect the five IO-board chips (3× PCA9634 RGB LED drivers, PCA9538A buttons, PCA9685 servo driver); rewrite set_rgb on raw HAL_I2C_Mem_Write; servo sweep. Short part: creating a .ioc in STM32CubeMX (board selection, pins and labels, peripherals, clock, NVIC), shown by rebuilding ass1.ioc; then duplicating it as the start of ass2.ioc. | A1 due |
| 6 | 7.12.2026 | PWM and input capture: duty cycle, H-bridge preview; SR04 trigger/echo by input capture; EXTI on the optical sensor (edge triggering revisited). | Blocking vs. non-blocking: writing a register inside the STM32 (one store, nanoseconds) vs. a register on another chip over I2C (about 0.3 ms); updating the 24 LED registers one per loop pass; the decide, store and transport layers and the super loop. SR04 single distance measurement; EXTI interrupt with a counter. |  |
| 7 | 14.12.2026 | DC motors and encoders: H-bridge drive, quadrature decoding (encoder timer mode), speed measurement, recording a step response at a fixed rate. | One wheel in the air: open-loop PWM, encoder to rad/s, record and export to MATLAB. Fit a model. | A2 out: PWM, polling and interups (sensors), I2C, encoders, step response to Matlab |
| 8 | 21.12.2026 | PI speed control: model fitting (V → rad/s), design, discretisation and sample rate, PI inside a 100 Hz interrupt, saturation and anti-windup, STM32CubeMonitor. | Different encoder configurations. | Last material included in A2 |
| 9 | 28.12.2026 | UART command protocol: interrupt-driven receive, command interpreter, ASCII vs. binary framing; MATLAB serialport and an App Designer GUI. | Control workflow: fit, sisotool, c2d, implement the difference equation in C, compare simulated vs. measured step. | Quiz 2 (first hour) |
| 10 | 4.1.2027 | Mecanum kinematics: inverse and forward kinematics, odometry; trajectory generation (trapezoidal profiles, waypoints, piecewise-constant (vx,vy,ω) with output-compare timing). | Kinematics (straight lines) and state machine: wheel speeds for driving in straight lines; a state machine (enum + switch) for a driving sequence. | A2 due |
| 11 | 11.1.2027 | Simulink to C: discrete blocks and sample rates, exporting a controller or filter, integrating it in CubeIDE; digital filters (median, IIR). | Export a Simulink PI/filter to C and run it on the cart. | A3 out: Controller design, obstacle avoiding, state machine |
| 12 | 18.1.2027 | System integration and case studies: scanned sonar map, obstacle response, wrap-up; final hints for A3. | Open project clinic. |  |
| 13 | 25.1.2027 | Lab meeting: open questions. |  | A3 due 3.3.2027 |

Last material included in A1/A2: an assignment may require material taught up to and including that lecture (one week after the assignment is out), and nothing taught later.

# Grading

| Component | Contents | Weight |
| --- | --- | --- |
| Quiz 1 | Sessions 1 to 3: logic and numbers, C types and casting, bit operations, IO board LEDs and buttons | 10% |
| Quiz 2 | Sessions 4 to 8: timers and interrupts, state machines, I2C, PWM and input capture, motors and encoders, PI speed control | 10% |
| Assignment 1 | Variable types, wrap-around, bit operations and shifting, logic, LED blinking, RGB LEDs | 20% |
| Assignment 2 | Software and hardware PWM, interrupts (EXTI, input capture), I2C drivers, encoders and speed, step response (wheels in the air) | 25% |
| Assignment 3 | PI speed control from the fitted model, driving in straight lines, obstacle avoidance with the IR and ultrasonic sensors, a state machine; bonus: return to the start | 35% |

# Assignments

## General instructions

- Report format: PDF only, digitally typed (not handwritten). The report may be written in Hebrew or English.

- Report structure:

  - Title page: assignment number, names of the submitters, ID numbers, emails and date.

  - An answer to each part. Do not Copy full codes section into the report, but DO explain what have you done (flow chart  pseudo code).

- Code: Use function for the code to be organized - each function must have a documentaion in the beginning - what it is suppose to do.
Document and explain your code, after a line of a code.
Write in the user-defined section, as taught.
Be oragnized. Unused codes section should not be included!

- Submission: a single ZIP file named lab#_studentname1_studentname2.zip, containing the code folders of the different parts, the report and a video demonstrating the system in operation.

- The grade is based, among other things, on the efficiency and clarity of the code.

- An engineering approach is expected: if you need to add a task, a code section, a state or a detail in the report in order to fulfil the requirements, do it. Creativity and elegant writing will be rewarded.

## Assignment 1: “No wheels”

Window: out at session 2, due at session 5. Hardware: NUCLEO-G474RE and the I2C IO board only. The motors are not powered. Allowed tools: HAL_Delay/HAL_GetTick, GPIO, printf over LPUART1, the debugger, and the provided IO-board functions. The I2C internals stay a black box until session 5.

Provided files. The project configuration is provided as ass1.ioc: you do not build it yourself (see “Assignment 1: project configuration” below; creating a .ioc is taught in tutorial 5). The RGB LEDs and the buttons sit on the I2C bus, which is taught only in session 5. Until then, use the provided files io_board.h and io_board.c: copy io_board.h into Core/Inc and io_board.c into Core/Src (in CubeIDE, refresh the project with F5 afterwards). In main.c, add #include "io_board.h" inside USER CODE BEGIN Includes (include the .h file only, never the .c file), and call ioBoardInit() once inside USER CODE BEGIN 2. CubeIDE compiles io_board.c automatically. To make printf print to PuTTY, copy the function in Appendix A1 into your main.c. [For Bucher and Yohai, delete before publishing: if the printf redirect to PuTTY is taught in lecture 3, perhaps this appendix can be deleted.]

Learning outcome. Students work confidently in C on real hardware. They choose variable types deliberately, predict and explain overflow and wrap-around, manipulate individual bits with shifts and masks, combine inputs with logic, and produce timed LED behaviour on the on-board LED and the RGB LEDs.

#### Part A: Variable types and wrap-around.

- Increment a uint8_t, an int8_t and a uint16_t past their limits. Watch them in Live Expressions. Print them to PuTTY in decimal, hex and binary, and capture it in a video (think how to do it so that the viewer can clearly see the wrap-around). In your report, describe what happens when a variable exceeds its limit, and explain why.

- Explain how to interpret a hex number as a decimal value, and the difference between the signed and the unsigned cases. Show examples for 0x7F (uint8_t and int8_t), 0xF0 (uint8_t and int8_t) and 0xFFF6 (uint16_t and int16_t). In the code, assign each value in its hex form to variables of both types, and print them to PuTTY in hex, binary and decimal. Explain what you see when a negative value is printed in hex.

- Integer division and casting. Two students took the MP course. One attended the classes and got a good grade, 97. The other did not, and got a bad grade, 58. Store each grade in an unsigned integer variable, then compute their average: first in a way that gives a wrong result because of integer arithmetic, then in a way that gives the correct result. Show both in the code, and explain the difference in your report.

#### Part B: Bit operations and shifting.

- Write the four bit idioms as functions (not macros). Each takes a value and a bit number, and returns the result: bitSet, bitClear and bitToggle return the modified value, and bitTest returns 0 or 1. For example: GPIOA->ODR = bitSet(GPIOA->ODR, 5); Use them to operate the Nucleo’s green LED (LD2, bit 5 of GPIOA->ODR) directly through the register, without HAL GPIO functions, in this sequence: 1 s of rest, SET; 1 s of rest, CLEAR; then 3 times (1 s of rest, TOGGLE); then TEST. Show the test result on LED 0 of the IO board: lit when the result is 1, off when it is 0.

- Walking single LED. Keep the state of the 8 RGB LEDs in one uint8_t variable, in which bit n represents LED n (1 = lit). Light one LED at a time, with 1 s between transitions: LED 0, then LED 1, and so on up to LED 7, then back to LED 0, forever. Print the variable in binary at every step.

- Walking pair of LEDs. The same, with two adjacent LEDs lit: LEDs 0 and 1, then 1 and 2, and so on; after LEDs 6 and 7 come LEDs 7 and 0, then 0 and 1 again.

Rules for B.2 and B.3: the variable may change only through bit operations (shifts, OR, AND). Choosing the pattern with if/switch chains, counters mapped to LEDs, or tables of precomputed patterns is not accepted. The LEDs are then lit from the variable, by testing its bits. In your report, explain how your code moves the pattern past bit 7 without losing it.

- Read the raw button byte with readButtonsRaw(). Using bit operations, convert it into a value in which bit n is 1 while button n is held: invert the active-low bits, then clear bits 4 to 7 with a mask. Count how many buttons are held (0 to 4) by testing each bit, and print the count whenever it changes. [For Bucher and Yohai, delete before publishing: io_board.h also provides readButtonsPressed(), which already returns (~raw) & 0x0F and so solves this item. Consider providing a copy of io_board without it.] [Also: setLedMask() in io_board.h lights LEDs straight from a bit mask, which shortens the display step of B.2 and B.3.]

#### Part C: Input triggering.

- Count presses of button 0 and show the count on the 8 RGB LEDs as an 8-bit binary number (bit n = LED n); print it as well. Do it three times:

  - Level: increase the counter on every pass of the main loop while button 0 is held. Describe what happens.

  - Edge: increase the counter only when button 0 changes from released to pressed (now & ~before), polling as fast as possible, with no delay in the loop. Press the button exactly 20 times and record the count. Repeat 3 times.

  - Debounced edge: accept a change of the button state only after it has been stable for 20 ms. Press exactly 20 times, 3 times over.

- In your report, compare the three results and explain each difference. [For Bucher and Yohai, delete before publishing: another bounce check will be done with the I2C bus at 400 kHz instead of the current 100 kHz.]

#### Part D: LED blinking and timing.

- Write a loop that toggles LD2, prints a status line of about 60 characters to PuTTY with printf, and then waits with HAL_Delay(500); stop after 200 toggles. Before running it, calculate how long the 200 toggles will take; then run it and measure. In your report, explain why the total is not exactly 200 × 500 ms, using the UART frame format and the baud rate.

- Blink LD2 (PA5) at a rate set by a global variable that can be changed in the debugger.

- Two LEDs blinking at different rates, done twice: first with HAL_Delay, then non-blocking with HAL_GetTick. Measure and explain the difference.

#### Part E: Integration and logic.

Write a program with an armed state that button 0 toggles (one press = one action, as in Part C). The trigger condition is: the system is armed and the IR sensor (PA7) detects an obstacle.

- While the condition holds, all 8 LEDs (white) grow brighter step by step, from off to full brightness, over about 2 s. When full brightness is reached, the LEDs switch off, and stay off until the condition has become false and true again.

- If the condition stops holding during the brightening, the LEDs dim step by step from their current brightness until they are off.

Set the brightness of all LEDs with setRGB(v, v, v) from io_board.h (v = 0 to 255). Pace the steps with HAL_GetTick, so the inputs are read on every pass of the loop. In your report, give the truth table of the trigger condition, and draw the behaviour as a state diagram. Creativity bonus applies here.

### Assignment 1: project configuration (provided: ass1.ioc)

The STM32CubeMX configuration for Assignment 1 is provided as ass1.ioc. The tables below list what it contains, for reference. The wheels are not used.

#### Creating the project.

You need STM32CubeIDE and STM32CubeMX.

- Create a folder named ass1 (for example inside your STM32CubeIDE workspace) and copy ass1.ioc into it.

- Open ass1.ioc in STM32CubeMX and click GENERATE CODE. The toolchain (STM32CubeIDE) is already selected. The project is generated into the ass1 folder.

- In STM32CubeIDE: File → Import → STM32CubeMX/STM32CubeIDE Project, and select the ass1 folder. Build the project once before adding your code.

[Yohai, delete before publishing: check these steps and the menu names on a clean PC with STM32CubeIDE 2.2 and STM32CubeMX 6.18 (in CubeIDE 2.x, CubeMX is a separate program).]

#### System.

| Item | Setting |
| --- | --- |
| MCU | STM32G474RET6 (Cortex-M4F with FPU) |
| System clock | SYSCLK = 170 MHz from the PLL (HSI 16 MHz source). HCLK = APB1 = APB2 = 170 MHz |
| Debug | Serial Wire: PA13 (SWDIO), PA14 (SWCLK), PB3 (SWO). Never reassign these pins. |
| Time base | SysTick, 1 ms (used by HAL_Delay and HAL_GetTick) |
| Board power | Power jumper on 5V_STLK (USB power) |

#### Pins.

| Pin | CubeMX signal | Label | Function |
| --- | --- | --- | --- |
| PA5 | GPIO_Output | LD2 | Green user LED on the Nucleo board |
| PC13 | GPIO_EXTI13 | B1 | Blue user button on the Nucleo board (EXTI, rising edge) |
| PA2 | LPUART1_TX |  | Serial console to the PC (ST-LINK virtual COM port) |
| PA3 | LPUART1_RX |  | Serial console from the PC |
| PB8 | I2C1_SCL |  | I2C clock to the IO board |
| PB9 | I2C1_SDA |  | I2C data to the IO board |
| PA7 | GPIO_Input, no pull | IR_SENSOR | IR obstacle sensor at the front of the cart. Active-low: LOW = obstacle |

#### Peripherals.

| Peripheral | Mode | Settings |
| --- | --- | --- |
| I2C1 | I2C | Standard mode, 100 kHz |
| LPUART1 | Asynchronous | 115 200 baud, 8 data bits, no parity, 1 stop bit |
| NVIC |  | EXTI line [15:10] interrupt enabled (B1) |

#### Devices on the I2C bus (IO board).

| Address (7-bit) | Chip | Role |
| --- | --- | --- |
| 0x01 | PCA9634 | Red channel of LEDs 0 to 7 (chip channel n = LED n) |
| 0x02 | PCA9634 | Green channel of LEDs 0 to 7 |
| 0x07 | PCA9634 | Blue channel of LEDs 0 to 7 |
| 0x71 | PCA9538A | Four push buttons on bits 0 to 3, active-low |

#### Serial terminal.

Connect PuTTY to the ST-LINK COM port at 115 200 baud, 8 data bits, no parity, 1 stop bit.

### Appendix A1: code to copy

#### printf to PuTTY.

Copy into main.c: the include into USER CODE BEGIN Includes, the function into USER CODE BEGIN 0.

#include <stdio.h>

/* route printf to LPUART1 (PuTTY) */
int __io_putchar(int ch)
{
    HAL_UART_Transmit(&hlpuart1, (uint8_t *)&ch, 1, HAL_MAX_DELAY);
    return ch;
}

End every printed line with \r\n: the output appears only when a line is complete.

## Assignment 2: “Wheels in the air”

Window: out at session 7, due at session 10. The assignment may use everything taught up to and including session 8. Hardware: the cart on its block, wheels in the air, battery and motor power on, the IO board, the IR sensor and the ultrasonic sensor. Project: ass2.ioc, which you build yourself from ass1.ioc as described in “Assignment 2: project configuration” below (no .ioc is provided for this assignment).

Provided files. io_board.h and io_board.c from Assignment 1 may still be used in Parts A, B, D and E. Part C replaces them with your own driver. The printf redirect of Appendix A1 is needed again.

Learning outcome. Students drive outputs with software and with hardware PWM and can explain the difference; they react to sensors by polling and by interrupts and can measure the advantage of interrupts; they write their own I2C driver from a datasheet; they configure an encoder timer, compute wheel speed in a way that survives counter wrap-around, and record a step response at a fixed rate for analysis in MATLAB.

Safety. The wheels must not touch anything while they turn. Keep fingers and cables away from the wheels, and set every PWM duty to 0 at start-up.

#### Part A: Three ways to make a PWM.

Build the same PWM with the three methods of tutorial 4 (polling, interrupt, hardware), with its symbols N, M, tstep and fPWM, and find the limits of each. N and M are global variables that you change from Live Expressions while the program runs.

Hint: keep two human limits in mind. The eye stops seeing a light flicker above roughly 50 to 100 Hz, and the ear hears from about 20 Hz to 20 kHz. They decide which PWM frequency is good enough for an LED and for a motor.

- Method 1, polling, on LD2, with HAL_GetTick as the step counter (tstep=1 ms). This task demonstrates a method that is not good enough, and why.

  - With N=100, sweep M from 0 to 100. What fPWM do you get, and what do you see?

  - Use M=N/2 and reduce N until the flickering disappears. Note N, fPWM and the number of duty levels that are left. Explain what you see and the trade-off you had to make.

  - Calculate the N needed for the 17 kHz PWM that TIM2 uses for the motors, and explain why it cannot be done with HAL_GetTick.

- Method 2, interrupt, with TIM7 at 10 kHz (tstep=100 μs, see the configuration section) and the same N and M as in A.1.

  - On LD2, with N=100: what fPWM do you get now, and does the LED flicker?

  - On wheel 1: in the callback, write 0 or 10 000 to TIM2->CCR1 (fully off or fully on). Try N=100 and N=10 at 50 % duty, and describe the motion and the sound of the wheel. [For Bucher, delete before publishing: what is the electrical time constant L/R of the cart’s motor windings? Can we write that it is about 1 ms? Then at 100 Hz (5 ms on, 5 ms off) the PWM is not fast enough compared with it: the current rises and falls fully in every period, the torque comes in pulses, and the motor is very loud.]

  - Try to make the motors’ PWM with method 2: 17 kHz with N=100. Which TIM7 settings (PSC, ARR) does it need, and would it work? Try it, and print the number of main-loop passes per second at 10 kHz and at the new rate.

  - Measure how long one update of an RGB LED takes over I2C (time 1000 calls of setLedChannel with HAL_GetTick). Explain why method 2 cannot drive the RGB LEDs from the TIM7 callback.

- Method 3, hardware, on wheel 1: TIM2 runs at 17 kHz with N=10 000, and TIM2->CCR1 sets M. Sweep the duty from 0 to 100 % in steps of 5 % and find the smallest duty at which the wheel starts to turn. On the RGB LEDs, set the brightness of LED 0 once with setLedChannel (the PCA9634 chip has its own hardware PWM). Count the main-loop passes per second again.

- Compare the three methods in a table: tstep, fPWM, N (duty levels), main-loop passes per second, and what you see and hear, on LD2, on the RGB LED and on the wheel. Explain which method suits an LED and which a motor, and why.

#### Part B: Polling vs. interrupts.

Wheel 1 turns at a constant duty of 40 %. It must stop as soon as the IR sensor detects an obstacle or the ultrasonic sensor measures a distance below 15 cm, and it may start again only after both have been clear for 1 s. The main loop also does the “rest of the program”: it updates all 8 RGB LEDs (green while the wheel turns, red while it is stopped), prints one status line, and then calls HAL_Delay(150) to stand for other work. Keep this workload in both versions.

- Without any interrupts. Disable the IR interrupt with HAL_NVIC_DisableIRQ(EXTI9_5_IRQn), read PA7 with HAL_GPIO_ReadPin, and start the echo capture with HAL_TIM_IC_Start (not the _IT version). Read each captured edge by polling the capture flag of TIM5 channel 2 (__HAL_TIM_GET_FLAG(&htim5, TIM_FLAG_CC2), then the captured value with HAL_TIM_ReadCapturedValue). Then test:

  - Move a hand quickly past the IR sensor, 10 times. How many times did the wheel stop?

  - Over 10 s, how many of the 100 ultrasonic measurements (one every 100 ms) gave a distance?

  - Measure the loop period with HAL_GetTick. From it, give the worst-case time from an obstacle appearing to the wheel stopping, and the angle the wheel turns in that time at 40 % duty (use your speed measurement from Part D or estimate it from the video).

- With interrupts. Use the EXTI interrupt of PA7 (both edges) and the TIM5 input-capture interrupt (HAL_TIM_IC_Start_IT). The callbacks stop the wheel themselves, by writing 0 to TIM2->CCR1, and set a flag; the main loop, with the same workload as in B.1, handles the LEDs and the restart. Repeat tests (a) to (c).

In your report, compare B.1 and B.2 in a table and explain the difference. Explain also why the callback may write TIM2->CCR1 but must not update the RGB LEDs, and which variables must be volatile. Show both versions in the video, with the wheel continuing to turn in B.1.

#### Part C: Your own I2C driver.

From now on the IO board is no longer a black box: do not use io_board.c in this part. Write your own functions on top of HAL_I2C_Mem_Write and HAL_I2C_Mem_Read, using the datasheets of the PCA9634, PCA9538A and PCA9685.

- Scan the bus: try every 7-bit address from 0x01 to 0x7F with HAL_I2C_IsDeviceReady and print the addresses that answer. Compare the list with the table of devices in the configuration section. In your report, explain what the STM32 sees on the bus when no chip has the address (ACK and NACK).

- LED driver. Write ledChipInit(addr) and ledSetChannel(addr, channel, value) for the PCA9634, setting the registers the datasheet requires (mode registers, output mode of each channel, PWM value). Use them to show a colour of your choice on each of the 8 RGB LEDs. In your report, list every register you write and why. Note how the LEDs are wired: check whether a larger PWM value makes an LED brighter or dimmer.

- One transaction or many. Set all 8 channels of one PCA9634 twice: once with 8 separate writes, and once with a single write of 8 bytes using the chip’s auto-increment (see the control register in the datasheet). Time 1000 repetitions of each. From the I2C bit rate (100 kHz) and the number of bits in each transaction (address, register, data, ACK bits, start and stop), calculate the expected times and compare them with your measurements.

- Buttons. Read the input register of the PCA9538A with HAL_I2C_Mem_Read and print the state of the 4 buttons whenever it changes. Check and print the return value of every HAL I2C call; show what is printed when you read from an address that has no chip.

- Servo. Initialise the PCA9685 for 50 Hz (PRESCALE = 121) and move the servo to 900, 1500 and 2100 μs. In your report, calculate the PWM frequency that PRESCALE = 121 gives, and the register values for each pulse width. [For Bucher and Yohai, delete before publishing: confirm on the board which PCA9685 channel the servo is on, and give it here.]

#### Part D: Encoders and wheel speed.

- Encoder resolution. The encoder timer of wheel 1 (TIM1) can count in Encoder Mode TI1 or in Encoder Mode TI1 and TI2. With the motor power off, mark the wheel and turn it by hand exactly 5 revolutions forward, then 5 back, and print the counter before and after. Do it with each of the two settings (change it in ass2.ioc and generate the code again). Give the counts per revolution and the resolution in degrees for each setting, and explain the factor between them from the A and B signals of a quadrature encoder.

- Speed and wrap-around. In the 100 Hz interrupt of TIM6, read the counter of TIM1 and compute the wheel speed from the change since the previous sample. The counter is 16 bits wide, so it wraps around (65 535 → 0 forward, 0 → 65 535 backward). Compute the change in two ways:

  - naively, as the difference of the two readings stored in int32_t variables;

  - as (int16_t)(now - previous), with now and previous of type uint16_t.

- Run the wheel at constant duty, forward and then backward, long enough for the counter to wrap several times. Record both speeds in rad/s and plot them against time. In your report, explain the spikes of (a), why (b) gives the right answer, and the highest wheel speed (in rev/s) for which (b) still works at 100 Hz, for each encoder setting of D.1. Relate it to the wrap-around of Assignment 1, Part A.

#### Part E: Step response to MATLAB.

- With wheel 1 at rest, step the duty from 0 to 30 % and record the speed of D.2(b) at 100 Hz for 2 s, into an array inside the TIM6 interrupt. Only after the recording ends, print the array to PuTTY as two columns (time in s, speed in rad/s). In your report, explain why the samples are stored first and printed afterwards, and not printed inside the interrupt.

- Save the PuTTY output to a file (PuTTY logging), load it into MATLAB and plot it. Repeat for steps to 50 % and 70 %, and plot the three responses on one figure.

- Fit a first-order model ω(t)=K u (1−e−t/τ) to each response. Give K and τ for each step size, and comment on whether the motor behaves linearly. Use the dead zone you found in A.4 in your explanation.

### Assignment 2: project configuration (you build ass2.ioc)

No .ioc file is provided for Assignment 2: you create ass2.ioc yourself, from ass1.ioc and the pinout and settings in this section. It contains the complete Assignment 1 configuration, plus the wheels (motors and encoders), the ultrasonic sensor and the servo that turns it. Do not start from scratch: duplicate ass1.ioc and add only the new items below.

- Open ass1.ioc in STM32CubeMX and save a copy with File → Save Project As into a new folder named ass2. This gives ass2.ioc; ass1.ioc stays unchanged.

- In ass2.ioc, add the pins and peripherals listed below. Leave the Assignment 1 settings as they are.

- Click GENERATE CODE, then import the ass2 folder into STM32CubeIDE as in Assignment 1.

All timer clocks are 170 MHz.

#### Pins from Assignment 1.

PA5 (LD2), PC13 (B1), PA2/PA3 (LPUART1), PB8/PB9 (I2C1), with the same settings as in Assignment 1. One change: PA7 (IR_SENSOR) becomes GPIO_EXTI7, trigger on rising and falling edges, no pull, label kept. It can still be read with HAL_GPIO_ReadPin.

#### Wheels: motors and encoders.

Each motor is driven by one PWM channel (speed) and one direction pin. Each encoder is read by its own timer in encoder mode.

| Motor | PWM pin | DIR pin | Encoder timer | Encoder pins (A, B) |
| --- | --- | --- | --- | --- |
| 1 | PA0 = TIM2_CH1 | PC2 (DIR1) | TIM1 | PC0 = TIM1_CH1, PC1 = TIM1_CH2 |
| 2 | PA1 = TIM2_CH2 | PB4 (DIR2) | TIM8 | PC6 = TIM8_CH1, PC7 = TIM8_CH2 |
| 3 | PB10 = TIM2_CH3 | PB11 (DIR3) | TIM3 | PA6 = TIM3_CH1, PA4 = TIM3_CH2 |
| 4 | PA10 = TIM2_CH4 | PB13 (DIR4) | TIM4 | PA11 = TIM4_CH1, PA12 = TIM4_CH2 |

The DIR pins are GPIO_Output, push-pull, no pull, initial level LOW. The level that drives a motor “forward” is SET for motors 1 and 4 and RESET for motors 2 and 3, because the motors on the two sides of the cart are mirrored.

#### Ultrasonic sensor (HC-SR04).

| Pin | CubeMX signal | Function |
| --- | --- | --- |
| PB2 | TIM5_CH1, PWM Generation CH1 | TRIG: a 10 μs pulse every 100 ms, generated by the timer |
| PC12 | TIM5_CH2, Input Capture direct mode | ECHO: both edges are captured; the pulse width is the round-trip time |

Distance: d [mm]=techo [μs]×343/2000.
The trigger pulses start only after HAL_TIM_PWM_Start(&htim5, TIM_CHANNEL_1).

#### Peripherals.

| Peripheral | Mode | Settings |
| --- | --- | --- |
| I2C1 | I2C | Standard mode, 100 kHz |
| LPUART1 | Asynchronous | 115 200 baud, 8N1 |
| TIM2 | PWM Generation CH1 to CH4 | Prescaler 0, period 9 999: 17 kHz PWM, duty = CCR / 10 000. Pulse 0, polarity high |
| TIM1, TIM8, TIM3, TIM4 | Combined channels: Encoder Mode TI1 | Prescaler 0, period 65 535: 16-bit up/down counter. Input polarity rising, filter 0 |
| TIM5 | CH1 PWM Generation, CH2 Input Capture direct mode | Prescaler 9, period 1 699 999: tick = 1/17 μs, period 100 ms. CH1 pulse = 170 (10 μs). CH2 polarity: both edges |
| TIM7 | Activated (basic timer) | Prescaler 169, period 99: 1 MHz tick, update interrupt at 10 kHz (the software PWM of A.2) |
| TIM6 | Activated (basic timer) | Prescaler 169, period 9 999: 1 MHz tick, update interrupt at 100 Hz (the sample rate of Parts D and E) |
| NVIC |  | EXTI line [9:5] (IR sensor), EXTI line [15:10] (B1), TIM2, TIM5, TIM6 (TIM6_DAC) and TIM7 (TIM7_DAC) global interrupts enabled |

#### Devices on the I2C bus.

As in Assignment 1, plus:

| Address (7-bit) | Chip | Role |
| --- | --- | --- |
| 0x44 | PCA9685 | Servo pulse generator, 50 Hz (PRESCALE = 121). Drives the servo that turns the ultrasonic sensor. Pulse width 900 μs (one end), 1500 μs (centre), 2100 μs (other end) |

## Assignment 3: “Wheels on the ground”

Window: out at session 11, due on 3.3.2027. The assignment may use everything taught up to and including session 12. Hardware: the cart on the floor, battery and motor power on, the IR sensor, the ultrasonic sensor on its servo, and three obstacles. Project: ass3.ioc, a copy of your ass2.ioc (File → Save Project As, as in Assignment 2). No new pins or peripherals are needed. [Bucher and Yohai: the obstacles (size, material that the ultrasonic sensor sees well, spacing along the path), the length of the course and the floor area in Room 26.]

Learning outcome. Students design a PI speed controller from their own plant model and run it on the cart; they drive the cart in straight lines from the wheel kinematics; they combine two sensors with different strengths into one decision and justify the strategy; they structure a driving task as a state machine.

Safety. Test at low speed first. B1 must stop all four wheels at any moment, in every state of your program. Keep the cart inside the test area and catch it if it drives towards people or furniture.

#### Part A: PI speed controller.

- Start from the first-order model you fitted in Assignment 2, Part E (K, τ). Choose a requirement for the closed loop (for example a settling time and the largest overshoot you accept), design a PI controller for the wheel speed that meets it, and give Kp and Ki. Explain your choice.

- Discretise the controller for the 100 Hz interrupt of TIM6 and implement it in C, for all four wheels. Limit the duty to its allowed range, and prevent integrator windup while the duty is saturated.

- With the wheels in the air, step the speed setpoint of wheel 1 and record the response as in Assignment 2, Part E. In MATLAB, plot the measured and the simulated closed-loop responses on one figure, and explain the differences.

- Repeat A.3 with the cart on the floor, all four wheels at the same setpoint. What changes, and why?

#### Part B: Obstacle course.

The cart drives forward along a straight line under the speed control of Part A. When it detects an obstacle ahead, it gets as close to it as it can without touching it, and stops. It bypasses the first obstacle from the right, as close to the obstacle as it can, and comes back to the original line. It bypasses the next obstacle from the left, and the third from the right again, each time returning to the original line and driving on.

- Use both sensors, the IR sensor and the ultrasonic sensor. Explain your strategy in the report: which sensor detects the obstacle, which one decides how close you get, what you do when they disagree, and how you handle false or missing readings. Support it with measurements of each sensor’s range and reliability.

- Write the program as a state machine (enum + switch). Include the state diagram in the report, with the condition for every transition.

- Use the encoders to measure how far the cart has moved. In the report, give the closest distance to each obstacle and the sideways distance from the original line after each bypass.

Show the whole course, all three obstacles, in the video.

#### Part C (bonus): Back to the start.

After the third obstacle, the cart returns to its starting point. Learn by yourself how to pivot the cart (turn it in place), and then drive back in straight lines. Report how far from the starting point the cart stopped, and explain where the error comes from. [Bucher and Yohai: the weight of the bonus, and whether the obstacles stay on the course for the way back.]

## Quizzes

Two quizzes, each 10% of the final grade, held in the first hour of the lecture: Quiz 1 in session 4 (23.11.2026), Quiz 2 in session 9 (28.12.2026).